/*

* 软件名称：无线倾角传感器监测系统(智能优化版)

* 硬件平台：ESP8266 + ADXL335

* 开发环境：Arduino IDE 2.3.2

* 系统版本：V2.0

* 主要功能：智能滤波、多级报警、云边协同、电源管理

*/

#include <ESP8266WiFi.h>

#include <ESP8266WebServer.h>

#include <ArduinoJson.h>

#include <EEPROM.h>

#include <ESP8266mDNS.h>

#include <WiFiUdp.h>

#include <ArduinoOTA.h>

#include <LittleFS.h>

// ========== 系统配置常量 ==========

#define OTA_PASSWORD "admin123"

#define CONFIG_VERSION 4

#define DEVICE_NAME "AngleMonitor"

// 硬件引脚定义

#define X_PIN A0

#define Y_PIN A1

#define Z_PIN A2

#define LED_PIN D4

#define BUZZER_PIN D5

// 系统参数

#define EEPROM_SIZE 512

#define TRANSMIT_INTERVAL 5000    // 数据传输间隔5秒

#define SAMPLE_BUFFER_SIZE 10     // 采样缓冲区大小

// ========== 枚举类型定义 ==========

enum AlarmLevel {

ALARM_NORMAL = 0,

ALARM_WARNING = 1,

ALARM_CRITICAL = 2,

ALARM_EMERGENCY = 3

};

enum PowerMode {

POWER_HIGH_PERF = 0,

POWER_BALANCED = 1,

POWER_SAVING = 2,

POWER_CRITICAL = 3

};

enum FilterType {

FILTER_NONE = 0,

FILTER_MOVING_AVG = 1,

FILTER_KALMAN = 2,

FILTER_HYBRID = 3

};

// ========== 数据结构定义 ==========

struct SystemConfig {

uint8_t version = CONFIG_VERSION;

// 网络配置

char sta_ssid[32] = "Your_WiFi_SSID";

char sta_pass[32] = "Your_WiFi_Password";

char ap_ssid[32] = "AngleMonitor_AP";

char ap_pass[32] = "12345678";

// 传感器参数

float offsets[3] = {0.0, 0.0, 0.0};

float scales[3] = {1.0, 1.0, 1.0};

float alarm_thresholds[3] = {15.0, 30.0, 45.0}; // 预警,报警,紧急

uint8_t calib_mode = 0;

uint8_t sample_rate = 10;

uint8_t filter_type = FILTER_HYBRID;

// 系统选项

bool serial_debug = true;

bool auto_calib = false;

bool enable_logging = false;

bool enable_ota = true;

bool use_ap_mode = false;

PowerMode power_mode = POWER_BALANCED;

};

struct SensorData {

float accel_x, accel_y, accel_z;

float temperature;

uint32_t timestamp;

};

struct AlarmStatus {

AlarmLevel level = ALARM_NORMAL;

float trigger_value = 0.0;

uint32_t start_time = 0;

bool acknowledged = false;

String message = "";

};

struct DeviceInfo {

char device_id[32] = "Default_Device";

char device_name[64] = "倾角监测设备";

char location[128] = "未设置位置";

uint32_t start_time = 0;

uint32_t last_online = 0;

};

// ========== 类定义 ==========

class AdaptiveFilter {

private:

float lpf_alpha = 0.7;

float kalman_Q = 0.001;

float kalman_R = 0.03;

float angle = 0.0;

float bias = 0.0;

float P[2][2] = {{0,0},{0,0}};

float last_lpf = 0.0;

public:

float processHybrid(float new_angle, float dt) {

// 低通滤波

float lpf_result = lpf_alpha * last_lpf + (1 - lpf_alpha) * new_angle;

last_lpf = lpf_result;

// 卡尔曼预测

angle += dt * (lpf_result - bias);

P[0][0] += dt * (P[1][1] * dt - P[0][1] - P[1][0] + kalman_Q * dt);

P[0][1] -= dt * P[1][1];

P[1][0] -= dt * P[1][1];

P[1][1] += kalman_Q * dt;

// 卡尔曼更新

float y = new_angle - angle;

float S = P[0][0] + kalman_R;

float K[2] = {P[0][0]/S, P[1][0]/S};

angle += K[0] * y;

bias += K[1] * y;

// 更新协方差

float P00_temp = P[0][0];

float P01_temp = P[0][1];

P[0][0] -= K[0] * P00_temp;

P[0][1] -= K[0] * P01_temp;

P[1][0] -= K[1] * P00_temp;

P[1][1] -= K[1] * P01_temp;

return angle;

}

float processMovingAverage(float new_angle) {

static float buffer[SAMPLE_BUFFER_SIZE] = {0};

static int index = 0;

static float sum = 0;

sum -= buffer[index];

buffer[index] = new_angle;

sum += buffer[index];

index = (index + 1) % SAMPLE_BUFFER_SIZE;

return sum / SAMPLE_BUFFER_SIZE;

}

void setParameters(float alpha, float Q, float R) {

lpf_alpha = alpha;

kalman_Q = Q;

kalman_R = R;

}

};

class AlarmManager {

private:

AlarmLevel current_level = ALARM_NORMAL;

unsigned long alarm_start_time = 0;

bool buzzer_state = false;

unsigned long last_buzzer_toggle = 0;

public:

AlarmStatus checkAlarm(float angle) {

AlarmStatus status;

status.trigger_value = angle;

if (abs(angle) >= config.alarm_thresholds[2]) {

status.level = ALARM_EMERGENCY;

status.message = "紧急！倾角超过安全阈值！";

} else if (abs(angle) >= config.alarm_thresholds[1]) {

status.level = ALARM_CRITICAL;

status.message = "报警！倾角较大！";

} else if (abs(angle) >= config.alarm_thresholds[0]) {

status.level = ALARM_WARNING;

status.message = "预警！倾角偏大";

} else {

status.level = ALARM_NORMAL;

status.message = "状态正常";

}

if (status.level > ALARM_NORMAL && current_level <= ALARM_NORMAL) {

status.start_time = millis();

alarm_start_time = status.start_time;

} else if (status.level > ALARM_NORMAL) {

status.start_time = alarm_start_time;

}

current_level = status.level;

return status;

}

void handleAlarmIndication(AlarmLevel level) {

if (level == ALARM_NORMAL) {

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

return;

}

// LED指示

digitalWrite(LED_PIN, HIGH);

// 蜂鸣器提示

unsigned long current_time = millis();

unsigned long interval = 0;

switch(level) {

case ALARM_WARNING: interval = 2000; break;

case ALARM_CRITICAL: interval = 1000; break;

case ALARM_EMERGENCY: interval = 200; break;

}

if (current_time - last_buzzer_toggle >= interval) {

buzzer_state = !buzzer_state;

digitalWrite(BUZZER_PIN, buzzer_state);

last_buzzer_toggle = current_time;

}

}

void acknowledgeAlarm() {

digitalWrite(BUZZER_PIN, LOW);

}

};

class PowerManager {

private:

PowerMode current_mode = POWER_BALANCED;

unsigned long last_mode_change = 0;

const unsigned long MODE_CHANGE_DELAY = 10000;

public:

void updatePowerMode(float battery_voltage, bool is_moving, bool network_activity) {

unsigned long current_time = millis();

if (current_time - last_mode_change < MODE_CHANGE_DELAY) return;

PowerMode new_mode = calculateOptimalMode(battery_voltage, is_moving, network_activity);

if (new_mode != current_mode) {

applyPowerMode(new_mode);

current_mode = new_mode;

last_mode_change = current_time;

if (config.serial_debug) {

Serial.printf("电源模式切换: %s\n", getPowerModeString(new_mode).c_str());

}

}

}

private:

PowerMode calculateOptimalMode(float battery_voltage, bool is_moving, bool network_activity) {

if (battery_voltage < 3.3) return POWER_CRITICAL;

if (battery_voltage < 3.6) return POWER_SAVING;

if (is_moving || network_activity) return POWER_HIGH_PERF;

return POWER_BALANCED;

}

void applyPowerMode(PowerMode mode) {

switch(mode) {

case POWER_HIGH_PERF:

WiFi.setSleepMode(WIFI_NONE_SLEEP);

system_update_cpu_freq(160);

break;

case POWER_BALANCED:

WiFi.setSleepMode(WIFI_LIGHT_SLEEP);

system_update_cpu_freq(80);

break;

case POWER_SAVING:

WiFi.setSleepMode(WIFI_MODEM_SLEEP);

system_update_cpu_freq(80);

break;

case POWER_CRITICAL:

WiFi.setSleepMode(WIFI_MODEM_SLEEP);

system_update_cpu_freq(40);

break;

}

}

String getPowerModeString(PowerMode mode) {

switch(mode) {

case POWER_HIGH_PERF: return "高性能";

case POWER_BALANCED: return "平衡模式";

case POWER_SAVING: return "节能模式";

case POWER_CRITICAL: return "临界模式";

default: return "未知";

}

}

};

class DataLogger {

private:

unsigned long last_log_time = 0;

const unsigned long LOG_INTERVAL = 60000; // 1分钟记录一次

public:

void logSystemStatus(float angle, AlarmLevel alarm_level, float battery_voltage) {

if (!config.enable_logging) return;

if (millis() - last_log_time < LOG_INTERVAL) return;

File file = LittleFS.open("/system_log.csv", "a");

if (file) {

String record = String(millis()) + ",";

record += String(angle, 2) + ",";

record += String(alarm_level) + ",";

record += String(battery_voltage, 2) + ",";

record += String(WiFi.RSSI()) + ",";

record += String(ESP.getFreeHeap());

file.println(record);

file.close();

last_log_time = millis();

if (config.serial_debug) {

Serial.println("系统日志记录: " + record);

}

}

}

};

// ========== 全局变量 ==========

SystemConfig config;

ESP8266WebServer server(80);

DeviceInfo device_info;

AlarmStatus alarm_status;

AdaptiveFilter angleFilter;

AlarmManager alarmManager;

PowerManager powerManager;

DataLogger dataLogger;

bool isMonitoring = false;

float currentAngle = 0.0;

float batteryVoltage = 3.7;

unsigned long lastTransmitTime = 0;

unsigned long systemStartTime = 0;

// ========== 系统核心函数 ==========

void loadConfig() {

EEPROM.begin(EEPROM_SIZE);

EEPROM.get(0, config);

EEPROM.end();

if(config.version != CONFIG_VERSION) {

config.version = CONFIG_VERSION;

saveConfig();

}

// 验证数据有效性

for(int i = 0; i < 3; i++) {

if(isnan(config.offsets[i])) config.offsets[i] = 0.0;

if(isnan(config.scales[i])) config.scales[i] = 1.0;

if(isnan(config.alarm_thresholds[i])) config.alarm_thresholds[i] = 15.0 + i * 15.0;

}

}

void saveConfig() {

EEPROM.begin(EEPROM_SIZE);

EEPROM.put(0, config);

EEPROM.commit();

EEPROM.end();

if(config.serial_debug) {

Serial.println("配置已保存");

}

}

SensorData readSensorData() {

SensorData data;

data.accel_x = analogRead(X_PIN) * 3.3 / 1023.0;

data.accel_y = analogRead(Y_PIN) * 3.3 / 1023.0;

data.accel_z = analogRead(Z_PIN) * 3.3 / 1023.0;

data.timestamp = millis();

// 应用标定参数

data.accel_x = (data.accel_x - config.offsets[0]) * config.scales[0];

data.accel_y = (data.accel_y - config.offsets[1]) * config.scales[1];

data.accel_z = (data.accel_z - config.offsets[2]) * config.scales[2];

return data;

}

float calculateAngle(float x, float y, float z) {

switch(config.calib_mode) {

case 1:  // Y轴模式

return atan2(z, sqrt(x*x + y*y)) * 180.0 / PI;

case 2:  // XY平面模式

return atan2(y, x) * 180.0 / PI;

default: // X轴模式

return atan2(y, z) * 180.0 / PI;

}

}

float applyFilter(float raw_angle, float dt) {

switch(config.filter_type) {

case FILTER_MOVING_AVG:

return angleFilter.processMovingAverage(raw_angle);

case FILTER_KALMAN:

return angleFilter.processHybrid(raw_angle, dt);

case FILTER_HYBRID:

return angleFilter.processHybrid(raw_angle, dt);

default: // 无滤波

return raw_angle;

}

}

void transmitToCloud() {

if (millis() - lastTransmitTime < TRANSMIT_INTERVAL) return;

// 模拟电池电压变化

batteryVoltage = 3.3 + (analogRead(A0) * 0.9 / 1023.0);

if (config.serial_debug) {

Serial.printf("云端传输: 角度=%.2f° 报警=%d 电池=%.2fV\n",

currentAngle, alarm_status.level, batteryVoltage);

}

lastTransmitTime = millis();

}

void initializeNetwork() {

Serial.print("连接到WiFi: ");

Serial.println(config.sta_ssid);

WiFi.begin(config.sta_ssid, config.sta_pass);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED && attempts < 20) {

delay(500);

Serial.print(".");

attempts++;

}

if (WiFi.status() == WL_CONNECTED) {

config.use_ap_mode = false;

Serial.println("\nWiFi连接成功!");

Serial.print("IP地址: ");

Serial.println(WiFi.localIP());

} else {

config.use_ap_mode = true;

WiFi.softAP(config.ap_ssid, config.ap_pass);

Serial.println("\nWiFi连接失败，启动AP模式");

Serial.print("AP IP: ");

Serial.println(WiFi.softAPIP());

}

}

// ========== Web页面处理函数 ==========

void handleRoot() {

String html = R"=====(

<!DOCTYPE html>

<html>

<head>

<meta charset="UTF-8">

<meta name="viewport" content="width=device-width, initial-scale=1">

<title>智能倾角监测系统</title>

<style>

* { box-sizing: border-box; margin: 0; padding: 0; }

body { font-family: 'Segoe UI', Arial, sans-serif; background: #f5f5f5; color: #333; }

.container { max-width: 1000px; margin: 0 auto; padding: 20px; }

.header { background: linear-gradient(135deg, #667eea 0%, #764ba2 100%); color: white; padding: 30px; border-radius: 15px; margin-bottom: 20px; box-shadow: 0 4px 15px rgba(0,0,0,0.1); }

.nav { display: flex; background: white; border-radius: 10px; padding: 10px; margin-bottom: 20px; box-shadow: 0 2px 10px rgba(0,0,0,0.1); }

.nav a { flex: 1; text-align: center; padding: 12px; text-decoration: none; color: #666; font-weight: 500; border-radius: 8px; transition: all 0.3s; }

.nav a:hover { background: #f0f0f0; color: #333; }

.nav a.active { background: #667eea; color: white; }

.card { background: white; border-radius: 15px; padding: 25px; margin-bottom: 20px; box-shadow: 0 4px 15px rgba(0,0,0,0.1); transition: transform 0.3s; }

.card:hover { transform: translateY(-2px); }

.btn { background: #667eea; color: white; border: none; padding: 12px 24px; border-radius: 8px; cursor: pointer; font-size: 14px; font-weight: 500; transition: all 0.3s; margin: 5px; }

.btn:hover { background: #5a6fd8; transform: translateY(-1px); box-shadow: 0 4px 12px rgba(102, 126, 234, 0.3); }

.btn-stop { background: #e74c3c; }

.btn-ack { background: #f39c12; }

.btn-stop:hover { background: #c0392b; }

.status-grid { display: grid; grid-template-columns: repeat(auto-fit, minmax(200px, 1fr)); gap: 15px; margin-bottom: 20px; }

.status-item { background: #f8f9fa; padding: 15px; border-radius: 10px; border-left: 4px solid #667eea; }

.angle-display { font-size: 3em; font-weight: bold; text-align: center; color: #667eea; margin: 20px 0; }

.alarm-normal { color: #27ae60; }

.alarm-warning { color: #f39c12; animation: pulse 2s infinite; }

.alarm-critical { color: #e74c3c; animation: pulse 1s infinite; }

.alarm-emergency { color: #c0392b; animation: pulse 0.5s infinite; background: #ffeaea; padding: 10px; border-radius: 8px; }

@keyframes pulse { 0% { opacity: 1; } 50% { opacity: 0.5; } 100% { opacity: 1; } }

.gauge { width: 200px; height: 200px; margin: 20px auto; }

</style>

</head>

<body>

<div class="container">

<div class="header">

<h1>🏗️ 智能倾角监测系统</h1>

<p>实时监测 • 智能报警 • 云端同步</p>

</div>

<div class="nav">

<a href="/" class="active">📊 实时监控</a>

<a href="/calibrate">🎯 传感器标定</a>

<a href="/settings">⚙️ 系统设置</a>

<a href="/diagnostic">🔧 系统诊断</a>

</div>

<div class="card">

<h2>📈 实时倾角监测</h2>

<div class="angle-display" id="angleDisplay">0.00°</div>

<div class="status-grid">

<div class="status-item">

<strong>报警状态</strong>

<div id="alarmStatus">正常</div>

</div>

<div class="status-item">

<strong>设备状态</strong>

<div id="deviceStatus">就绪</div>

</div>

<div class="status-item">

<strong>电池电量</strong>

<div id="batteryStatus">--%</div>

</div>

<div class="status-item">

<strong>运行时间</strong>

<div id="uptimeStatus">--</div>

</div>

</div>

<div style="text-align: center; margin-top: 20px;">

<button class="btn" onclick="sendCommand('start')">▶️ 启动监测</button>

<button class="btn btn-stop" onclick="sendCommand('stop')">⏹️ 停止监测</button>

<button class="btn btn-ack" onclick="sendCommand('acknowledge')">🔕 确认报警</button>

</div>

</div>

<div class="card">

<h3>📋 报警阈值设置</h3>

<div style="display: flex; justify-content: space-around; text-align: center;">

<div>

<div style="color: #f39c12; font-size: 1.2em;">⚠️ 预警</div>

<div id="threshold1">15.0°</div>

</div>

<div>

<div style="color: #e74c3c; font-size: 1.2em;">🚨 报警</div>

<div id="threshold2">30.0°</div>

</div>

<div>

<div style="color: #c0392b; font-size: 1.2em;">💥 紧急</div>

<div id="threshold3">45.0°</div>

</div>

</div>

</div>

</div>

<script>

function updateAlarmDisplay(level, message) {

const alarmElem = document.getElementById('alarmStatus');

alarmElem.textContent = message;

alarmElem.className = '';

switch(level) {

case 1: alarmElem.className = 'alarm-warning'; break;

case 2: alarmElem.className = 'alarm-critical'; break;

case 3: alarmElem.className = 'alarm-emergency'; break;

default: alarmElem.className = 'alarm-normal';

}

}

function sendCommand(cmd) {

fetch('/control?action=' + cmd)

.then(response => response.text())

.then(data => console.log('Command result:', data))

.catch(err => console.error('Command error:', err));

}

function updateDisplay() {

fetch('/data')

.then(r => r.json())

.then(data => {

// 更新角度显示

const angleElem = document.getElementById('angleDisplay');

angleElem.textContent = data.angle.toFixed(2) + '°';

angleElem.className = 'angle-display ' + getAlarmClass(data.alarm_level);

// 更新报警状态

updateAlarmDisplay(data.alarm_level, data.alarm_message);

// 更新设备状态

document.getElementById('deviceStatus').textContent =

data.monitoring ? '监测中' : '已停止';

// 更新电池状态

document.getElementById('batteryStatus').textContent =

data.battery_level + '% (' + data.battery_voltage + 'V)';

// 更新运行时间

document.getElementById('uptimeStatus').textContent =

formatUptime(data.uptime);

})

.catch(err => {

console.error('Update error:', err);

document.getElementById('deviceStatus').textContent = '连接错误';

});

}

function getAlarmClass(level) {

switch(level) {

case 1: return 'alarm-warning';

case 2: return 'alarm-critical';

case 3: return 'alarm-emergency';

default: return 'alarm-normal';

}

}

function formatUptime(seconds) {

const hours = Math.floor(seconds / 3600);

const minutes = Math.floor((seconds % 3600) / 60);

return hours + '时' + minutes + '分';

}

// 初始化显示阈值

document.getElementById('threshold1').textContent = )=====" + String(config.alarm_thresholds[0]) + R"=====( + '°';

document.getElementById('threshold2').textContent = )=====" + String(config.alarm_thresholds[1]) + R"=====( + '°';

document.getElementById('threshold3').textContent = )=====" + String(config.alarm_thresholds[2]) + R"=====( + '°';

// 每500ms更新数据

setInterval(updateDisplay, 500);

updateDisplay();

</script>

</body>

</html>

)=====";

server.send(200, "text/html", html);

}

void handleData() {

DynamicJsonDocument doc(1024);

doc["angle"] = currentAngle;

doc["monitoring"] = isMonitoring;

doc["alarm_level"] = alarm_status.level;

doc["alarm_message"] = alarm_status.message;

doc["alarm_acknowledged"] = alarm_status.acknowledged;

doc["battery_level"] = constrain((int)((batteryVoltage - 3.0) / 1.2 * 100), 0, 100);

doc["battery_voltage"] = String(batteryVoltage, 2);

doc["uptime"] = (millis() - systemStartTime) / 1000;

doc["device_id"] = device_info.device_id;

doc["device_name"] = device_info.device_name;

doc["free_heap"] = ESP.getFreeHeap();

doc["wifi_rssi"] = WiFi.RSSI();

String json;

serializeJson(doc, json);

server.send(200, "application/json", json);

}

void handleControl() {

if (server.hasArg("action")) {

String action = server.arg("action");

String response = "";

if (action == "start") {

isMonitoring = true;

response = "监测已启动";

} else if (action == "stop") {

isMonitoring = false;

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

response = "监测已停止";

} else if (action == "acknowledge") {

alarmManager.acknowledgeAlarm();

alarm_status.acknowledged = true;

response = "报警已确认";

} else {

response = "未知命令";

}

server.send(200, "text/plain", response);

if (config.serial_debug) {

Serial.println("控制命令: " + action + " - " + response);

}

}

}

void handleCalibrate() {

String html = R"=====(

<!DOCTYPE html>

<html>

<head>

<meta charset="UTF-8">

<title>传感器标定</title>

<style>

body { font-family: Arial; max-width: 600px; margin: 0 auto; padding: 20px; }

.calib-step { background: #e3f2fd; padding: 15px; margin: 10px 0; border-radius: 8px; }

.btn { background: #2196F3; color: white; padding: 10px 20px; border: none; border-radius: 4px; cursor: pointer; }

</style>

</head>

<body>

<h1>传感器标定</h1>

<p>请按照步骤完成传感器标定：</p>

<div class="calib-step">

<h3>步骤1: 水平放置</h3>

<p>将设备水平放置，点击开始采集</p>

<button class="btn" onclick="startCalibration(0)">开始采集</button>

</div>

<div class="calib-step">

<h3>步骤2: 垂直放置</h3>

<p>将设备垂直放置，点击开始采集</p>

<button class="btn" onclick="startCalibration(1)">开始采集</button>

</div>

<button class="btn" onclick="location.href='/'">返回首页</button>

<script>

function startCalibration(step) {

fetch('/calibrate?step=' + step)

.then(r => r.text())

.then(data => alert('标定步骤 ' + step + ' 完成: ' + data));

}

</script>

</body>

</html>

)=====";

server.send(200, "text/html", html);

}

void handleDiagnostic() {

String html = R"=====(

<!DOCTYPE html>

<html>

<head>

<meta charset="UTF-8">

<title>系统诊断</title>

<style>

body { font-family: Arial; max-width: 600px; margin: 0 auto; padding: 20px; }

.diagnostic-item { background: #f5f5f5; padding: 15px; margin: 10px 0; border-radius: 8px; border-left: 4px solid #4CAF50; }

</style>

</head>

<body>

<h1>系统诊断信息</h1>

<div class="diagnostic-item">

<h3>设备信息</h3>

<p>设备ID: )=====" + String(device_info.device_id) + R"=====(</p>

<p>设备名称: )=====" + String(device_info.device_name) + R"=====(</p>

<p>运行时间: <span id="uptime">计算中...</span></p>

</div>

<div class="diagnostic-item">

<h3>系统状态</h3>

<p>内存空闲: <span id="memory">)=====" + String(ESP.getFreeHeap()) + R"=====( bytes</span></p>

<p>Flash大小: )=====" + String(ESP.getFlashChipSize()) + R"=====( bytes</p>

</div>

<div class="diagnostic-item">

<h3>网络状态</h3>

<p>连接模式: )=====" + (config.use_ap_mode ? "AP模式" : "STA模式") + R"=====(</p>

<p>IP地址: )=====" + (config.use_ap_mode ? WiFi.softAPIP().toString() : WiFi.localIP().toString()) + R"=====(</p>

<p>信号强度: )=====" + String(WiFi.RSSI()) + R"=====( dBm</p>

</div>

<button class="btn" onclick="location.href='/'">返回首页</button>

<script>

function updateUptime() {

fetch('/data')

.then(r => r.json())

.then(data => {

document.getElementById('uptime').textContent =

Math.floor(data.uptime / 3600) + '时' + Math.floor((data.uptime % 3600) / 60) + '分';

});

}

setInterval(updateUptime, 5000);

updateUptime();

</script>

</body>

</html>

)=====";

server.send(200, "text/html", html);

}

void handleNotFound() {

server.send(404, "text/plain", "页面未找到");

}

// ========== 主程序 ==========

void setup() {

Serial.begin(115200);

Serial.println("\n\n=== 智能倾角监测系统启动 ===");

// 初始化硬件

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

digitalWrite(BUZZER_PIN, LOW);

// 加载配置

loadConfig();

// 初始化文件系统

if (LittleFS.begin()) {

Serial.println("LittleFS初始化成功");

} else {

Serial.println("LittleFS初始化失败");

}

// 设置设备信息

String chipId = String(ESP.getChipId());

strncpy(device_info.device_id, ("Angle_" + chipId).c_str(), sizeof(device_info.device_id));

device_info.start_time = millis();

systemStartTime = millis();

// 网络连接

initializeNetwork();

// OTA更新

if (config.enable_ota) {

ArduinoOTA.setPassword(OTA_PASSWORD);

ArduinoOTA.begin();

Serial.println("OTA更新服务已启动");

}

// mDNS服务

if (MDNS.begin("anglesensor")) {

Serial.println("mDNS服务已启动: anglesensor.local");

}

// Web服务器路由

server.on("/", handleRoot);

server.on("/data", handleData);

server.on("/control", handleControl);

server.on("/calibrate", handleCalibrate);

server.on("/diagnostic", handleDiagnostic);

server.onNotFound(handleNotFound);

server.begin();

Serial.println("HTTP服务器已启动");

Serial.println("系统初始化完成，等待命令...");

}

void loop() {

server.handleClient();

ArduinoOTA.handle();

if (isMonitoring) {

static unsigned long lastSampleTime = 0;

unsigned long currentTime = millis();

float dt = (currentTime - lastSampleTime) / 1000.0;

if (currentTime - lastSampleTime >= (1000 / config.sample_rate)) {

// 读取传感器数据

SensorData sensorData = readSensorData();

// 计算原始角度

float raw_angle = calculateAngle(sensorData.accel_x, sensorData.accel_y, sensorData.accel_z);

// 应用滤波

currentAngle = applyFilter(raw_angle, dt);

// 检查报警

alarm_status = alarmManager.checkAlarm(currentAngle);

// 处理报警指示

alarmManager.handleAlarmIndication(alarm_status.level);

// 数据传输

transmitToCloud();

// 电源管理

bool is_moving = (abs(raw_angle - currentAngle) > 0.5);

powerManager.updatePowerMode(batteryVoltage, is_moving, true);

// 数据记录

dataLogger.logSystemStatus(currentAngle,